Download firmware, programs and IP-address to PLC with SD-memory card.
[image: image1.png]


Mount SD-card in card slot on PLC and switch on the power, wait until the run indication flashes slowly.
Switch of the power and remove the SD-card to finish.
[image: image2.png]HEERERIRESEEE
Logaing
Show Log at Boot

Show system ssttings

About,




Run indication
[image: image3.png]


SD-card slot
Download firmware and program to HMI with USB flash drive.


Mount USB flash drive to the USB slot and switch on the power.
Press anywhere on panel until the system menu appears.
System menu.
[image: image4.png]Info

Name: UN31
Serial: 0x000000
Date: 27-09-11
HW: 68.68.009F

Plugin List




[image: image5.png]




Press update.
[image: image6.jpg]





Select “Auto select best match”.

[image: image7.png]ZoomIn
Zoom Out
Zoom 100%

Pan mode

Sattings.
Project Manager.

Update.

Backup/Restore.
Logaing
Show Log at Boot

Show system ssttings

About,

Figure 251
The wizard uilty will start. Check "Auto select best match’. and then click "Next"

HMI Update wizard 1/2

Flease walt, examining system.

Avalble updates: [\Usrermory <]

Auto select best match

Components that wil be updated;

Froject Fles
Fort fles
System Fies

Next cancel







Press next.

The update is ready when the panel has rebooted.

Press anywhere on panel until the system menu appears.


                                Press Show system settings.    


  Press Network


                                                                                                   Select Specify an IP address

                                                                                                  Use the keyboard to specify 193.168.1.197

                                                                                                                                                255.255.255.0

When ready close the dialog with “OK” and then “X” and move on to  
”Time” and select Time zone GMT +/- 0 (London).
Make sure that ”Automatically adjust clock for daylight saving” not is selected.
Press ”Apply”.
Leave system settings by pressing “X” in the upper right corner.


Press the upper left corner for at least 15 seconds to load 
all default values.

Complete installation by select Mode (Prover, Retarder-prover or Freezer-prover) and language.
130918 
 2(2)

